SCOPE OF WORK

- SCOPE OF WORK: Supply, Fabrication, Erection, Surface treatment and Testing of the SS-316L Acid Day Tank for leaching as per drawing no. UCT-2/CHEM/TANK/98 on constructed civil foundations at Turamdih Mill as per the detail given below:

- Supply of Steel materials like SS-316L Plates, Angles, Channels etc. of only SAIL/ JINDAL make required for complete work as per drawing enclosed with tender document. The Plate shall be of minimum 1.25 mtr wide x 5.0 m long. Other items shall be supplied in standard lengths.

- Supply of all consumables like Welding & cutting electrodes, gas, grinding / cutting wheel of approved make of UCIL.

- Only Argon Arc process shall be used for all welding.

- Arranging all tools and tackles, safety items, ladders, scaffolding etc. required for the completion of the job.

- Arranging for all testing instruments and tools (like dye penetration test) for checking of welding quality.

- Unloading of the material with your own arrangement at our site shown by the Corporation.

- Arranging for plate bending machine and transport facility for shifting of man and material.

- Fabrication of the tanks, tank covers, nipples for inlet/ outlet/ drain/ overflow/ instrument mounting points etc. manholes, cover fixing supports, patch plates for supporting walkways, other dosing and maintenance arrangement inside the tanks etc. as per the supplied drawing and the site decision of the Engineer-in-charge. All nozzles specified in the drawing will be one end welded and other with Standard SS-316L Flanges. Nozzle projection will be as per drawing.

- Any other changes will be as per Minutes of Meetings & subsequent correspondences.

- Removal of all scraps separately to designated area within 500 mtr range as directed by the Engineer-in-charge.

NOTE:

- Tank shall be fabricated at UCIL site only.

- Supplied stainless steel material shall be tested by UCIL at our laboratory only and report shall be final & binding to you. In case of any dispute material may be tested at your cost from a NABL certified laboratory.

- Any other items or work not specifically mentioned above but required for the completion of fabrication, surface treatment and testing of the tanks will have to be arranged by bidder with no extra payment.

- The tank shall be guaranteed for satisfactory performance for one year from the date of taking over. Any defect generated or noticed during one year from the date of handing over will have to be rectified by the bidder with no extra cost within 30 days of the information. In case of not rectifying the defect within the stipulated time, Corporation shall get the defect rectified at risk and cost of the supplier with no further correspondence.

- The bidder shall have to submit program for completion of the work and get it approved from Engineer-in-charge within 15 days from the date of receipt of Purchase Order.

- Fabrication scrap will be the property of the Corporation.

- Single Point electricity and water will be provided at UCIL site free of cost.

- Schedule of Work

- You will have to complete all the work strictly within four months from the date of LOI/P.O.

- You should supply steel items as per scope of work for the full job within 30 days of the order.

- SCRAP REMOVAL

Removal of all scraps separately to designated area within 500 mtr range as directed by the Engineer-in-charge.

- MAKE OF MATERIAL:

| Description of Material | Make | Specification |
| --- | --- | --- |
| SS-316L plates, angles, channel | SAIL /JINDAL | SS-316L |
| SS fasteners | Agrawal fasteners/ Finolex / Laxmi fasteners / Deepak fasteners | SS-316L |
| SS pipes | M/s Modern tubes/ M/s Quality steel/M/s remi pipes & fittings. | SS-316L |
| Structural items | SAIL /TISCO/JINDAL | IS:2062 Grade A |
| Handrail Pipe | Nezone /Bansal /Tata /Jindal | IS:1239 (Medium Class) Part-I |
| Paint | ICI (dulox synthetic enamel) | 1st quality paint |
|  | BERGER(luxol hygloss) |  |
|  | ASIAN PAINTS (apcolite premier gloss enamel) |  |

- SS-316 plates/angle/channels shall have stamping for batch number, lot number, heat number, MOC, manufacturer name etc.

- Party shall submit material test certificate from the manufacturer.

- Material shall be tested at UCIL’s laboratory for conformation of MOC.

6.  TEST REPORT/DOCUMANTATION:

Party shall submit the following documents

- Material purchase certificate for all steel items from manufacturer or authorized dealer.

- Paint purchase certificate from manufacturer or authorized dealer.

- Inspection / test reports of bought out items.